Flight Ops Instruction No  015




v1.2   29 Aug 2013

ATS Oceanic Communication beyond NZFIR in ZK-PLK

Given this IFR aircraft has one Collins HF and one DZM Flightcell Iridium Satellite Phone as well as 2 VHF radios, ATS communication beyond NZFIR is to be carried out initially by VHF, then HF and if needed a backup of the Sat phone.

All flight crew are to be trained and certified as competent in the use of all communication equipment.

Oceanic Communication Procedure:

1. Include in Telephone Index:  To file a flight plan by Cellphone 0800-626756



  
      To file a flight plan by Sat phone 0064-3-3581509



                  Inflight Oceanic ATS calls use 0064-9-2759817

2. Advise ATS of Aircraft ZK-PLK Sat Phone Number 0088-165-144-8383.
3.  When filing an Oceanic flight plan operating in NZZO FIR include  the aircraft Satellite Phone number in field 18 of the flight plan. 
4. Pre-Takeoff – VHF, HF, and Sat Phone check all tested and serviceable
    before t/o.
5. Review communication failure procedures in the Pink Emergency section of the AIP and the procedures below: 

    
In the event of HF failure enroute –



a) Use Sat phone to maintain two-way communications with 



    Oceanic Control.



b) Monitor both 123.45mhz and 121.5mhz VHF channels



c) Use standard reporting and enroute procedures at all times.

6.  Use VHF in departure and change to HF when instructed by ATS. 


1

