ieX FOI 134 V1.0
09 Dec 08 


FLIGHT OPERATION INSTRUCTION – FOI 134
 Beaufort 5 Person Life Raft
1. Introduction
The Beaufort 6/5 person life raft can be carried both in the fixed wing and rotary wing aircraft during over water operations.  It is designed for 5 persons normally but has an overload capacity of 6 persons. It is important that all flight crew are familiar with its contents, how to store and operate it, and what it looks like both stowed and inflated.  This FOI is designed to give clear instructions on how to active this.  The life raft should be securely stowed next to the aircraft exit area.
2. Photos of Life Raft
[image: image1.jpg]


     [image: image2.jpg]



Beaufort 6/5 person life raft



Deployment Line

[image: image3.jpg]


     [image: image4.jpg]





Inflated





Survival contents
3.
Life Raft Equipment List

The following equipment is stowed inside the life raft.

· T.R.O. (glue & patches)

· Torch including 4 x batteries & spare bulb

· Bellows

· Bailer

· Leak stoppers

· Paddles

· Signal card

· 3 x 500 mls drinking water

· First aid kit
· Survival booklet

· Drinking cup

· 2 x day/night flares

· 2 x parachute flares

· Anti sea sickness tablets

· Heliograph
· Sea sickness bags

· Scissors

Note:  The life raft does not contain an emergency locator transmitter (ELT). 

4.
Inflation of the Life raft
One designated crewperson should be made responsible for exiting the aircraft and deploying the life raft.  The life raft is inflated by pulling on the 7 metre long lanyard located under a flap secured by velcro which activates a firing device on a CO2 cylinder. The life raft has a self inflating canopy as well as self activating external and internal lights.  Inside the life raft you will find an orange bag attached to the floor which contains numerous items (see life raft equipment list). 
Note: NEVER inflate the life raft inside the aircraft cabin.

5.
Inflation Procedure

· Designated crewperson will exit the aircraft with the life raft being careful not to damage the life raft on sharp surfaces. 

· Gather survivors together next to life raft and ensure well clear of aircraft and/or wreckage to avoid possible puncture of life raft on inflation.
· Inflate life raft by pulling on the 7 metre lanyard located behind the velcro flap ensuring lanyard line is held securely by survivors to avoid life raft possibly being blowing away by wind or sea conditions.
· Once fully inflated survivors to grab hold of life lines on outside of raft.
· Remove shoes and sharp objects prior to boarding to avoid damage to raft

· First survivor boards raft while others remain holding onto life lines.  Once first person is aboard they help other survivors aboard one at a time. 
· Familiarise with survival equipment and follow immediate instructions. 
· Use paddles as required to keep raft away from aircraft and wreckage which may puncture raft.
· Secure canopy. 

· Continue to follow life raft instructions.
Note:  Upon complete inflation of the life raft a leaking sound may be heard for some time after inflation is complete.  This is normal and is simply excess CO2 escaping through a relief valve.

PAGE  
1

