[image: image1.jpg]SicrLINE

C
AViIATION



   B200  Operational Checklist
                                     13 May 2014          10.2.3.1  

	

	Pre-Start Checklist – From the line
	

	Cabin Door
	Considered

	Seats and Belts
	Secure

	Park Brake
	Set

	Fuel Quantity          (Mains & Aux)
	Checked

	Engine Anti-ice
	On

	Annunciators
	Normal

	Battery/Voltmeter
	On/23VCtr

	Power Levers
	Idle

	Props
	High RPM

	Condition Levers
	Cut Off

	Bleed Air
	Closed

	Rotating Beacon & Nav Lights
	On

	
	

	After Start Checklist
	

	DC Voltage and Load
	Check

	Medical Power
	On

	Temp Mode/Blowers/Bleed Air
	Auto/As Req

	Inverter
	On

	Avionics Master 
	On

	Lights/Cabin signs
	As Req

	Annunciators
	Consider

	Engine Instruments
	Checked

	
	

	Taxi Checklist
	

	Brakes
	Check

	Flight Instruments & Avionics
	Check

	Flight Director
	GA &ALTSEL

	Nav Aids
	Tuned/ID

	Trim Tabs
	Set as Req

	Flaps 
	Set 

	Pressurisation
	Set 

	ATC clearance/Dep Brief/Alt Alert
	Briefed/Set

	T/O Brief
	Briefed

	Inst Vacuum/pneumatics/De Ice
	Checked

	Flight Controls
	Full & Free

	Prop Manual Fx
	Checked

	
	

	Line-Up Checklist
	

	Auto feather
	Armed

	Ice Vanes
	As req

	Igniters
	If req, Arm

	Props
	Full Forward

	Transponder
	Alt

	Strobes
	On

	Annunciators
	Normal 

	Cabin
	Clear/Secure

	Condition Levers
	High Idle

	Ice Protection & Landing Lights
	On (Cld  T/O)

	
	

	After Take-Off Checklist
	

	Gear
	Up, No Lights

	Flaps
	Up/Indicating

	Climb Power      (Props 1900rpm)
	Set

	Pressurisation
	Checked

	Landing & Taxi Light
	Off

	Ice Vanes
	As Req

	Engine instruments
	Checked

	YD and Autopilot
	Engaged

	Ground Transport
	Advise ETA


	Transition Checks
	Passing FL130

	Altimeters
	Set 1013Hpa

	Recogs
	Off

	Pressurisation
	Checked

	Altitude Alerter
	Set

	
	

	Cruise Checklist
	

	Cruise Power 
	Set

	Autofeather
	Off

	Engine Instruments
	Checked

	Pressurisation
	Checked

	Fuel Quantity
	Checked

	
	

	Descent/Approach Checklist
	

	Alt Alert
	Set

	Fuel
	Checked

	Auto Feather
	Armed

	Pressurisation
	Set

	Approach Brief/VREF/QNH
	Reviewed

	Navaids
	Set/Tuned

	Windshield Heat
	On/Off

	Cabin Signs / Lights / PA
	As Req

	
	

	Transition Checks
	Passing FL150

	Altimeters
	Set QNH

	Recogs
	On

	Pressurisation
	Checked

	Altitude Alerter
	Set

	
	

	Landing Checklist 
	(once gear down)

	Brakes / Park Brake
	Pressure / Off 

	Landing Gear
	Down, 3 Greens

	Flaps
	Set Approach

	Ice Vanes
	On

	
	

	Finals Check (Memory Items
	Once Flap LDG)

	
	

	After Landing Checklist
	

	Engine Anti-Ice
	On

	Auto Ignition
	Off

	Lights/Strobes
	Off

	Ice Protection
	Off

	Transponder
	GND/Standby

	Flaps
	Up/Indicating

	Trims
	Set T/O x3

	Radar
	Standby & Up

	
	

	Before Shutdown Checklist
	

	Park Brake
	On

	Avionics Master
	Off

	Inverter
	Off

	Medical Power
	Off

	Blowers / Mode Controller
	Off

	Bleed Air
	Off

	Oxygen
	Off

	Battery
	Charged

	ITT’s
	Stabilised (2 mins)

	Condition & Prop Levers
	Cut-off /Feathered

	DC Volt/Load
	Check

	Gang Bar
	Off



[image: image1.jpg]